IR receiver

Membrane keypad

Code

- For the testing code (to slightly adapt from 4x4 to 4x3): https://www.instructables.com/id/Connecting-a-4-x-4-Membrane-Keypad-to-an-Arduino/

Flexible membrane

- Piece of code to modify:

  - // MEMBRANE KEYPAD FLEXIBLE

  - byte rowPins[ROWS] = {9, 8, 7, 6}; //connect to the row pinouts of the keypad

  - byte colPins[COLS] = {5, 4, 3}; //connect to the column pinouts of the keypad

- Picture:

From https://www.addicore.com/v/vspfiles/downloadables/Product%20Downloadables/Project_Interface_Kit/Addicore_12-Key_Keypad_Tutorial.pdf

None-flexible membrane

- Resource for graph and code: http://learning.grobotronics.com/2013/07/using-a-3x4-keypad/

- Piece of code to modify:

  - // MEMBRANE KEYPAD HARD

  - byte rowPins[ROWS] = {5, 4, 3, 2}; //connect to the row pinouts of the keypad

  - byte colPins[COLS] = {8, 7, 6}; //connect to the column pinouts of the keypad

- Pictures:

RTC module

- Model: Tiny RTC module DS1307

- Only for 5V

- Good resource in French: https://www.carnetdumaker.net/articles/utiliser-un-module-horloge-temps-reel-ds1307-avec-une-carte-arduino-genuino/#le-module-ds1307

- Connections :

  - VCC --> +5v

  - GND --> GND

  - SDA --> pin A4

  - SCL -->pin A5